以下配置针对ros2 humble，ubuntu22.04系统（ros2 humble通过fishros一键安装即可，请务必也在无conda等虚拟环境的情况下安装ros2）

以下教程建议全程禁用 conda 及其他虚拟环境.特别提醒：即使是conda安装后自动启动的base环境也不可以，使用conda deactivate退出base环境。

请先观看《基于ardupilot的gazebo仿真环境配置教程.docx》，配置好ardupilot，gazebo，ardupilot_gazebo插件以后再看此教程

0．退出虚拟环境

bash

conda deactivate conda config --set auto_activate false

使用命令行conda config --set auto_activate false配置 conda 不自动激活 base，这样不用每次开新终端时都执行conda deactivate。建议：不仅配置环境的时候不要使用虚拟环境，运行仿真的时候也不要使用虚拟环境。（特别是conda会对ros2 gazebo等这些硬件相关的工具的环境变量造成巨大的篡改，导致难以解释和解决的错误；即使是venv也很费事，为了简单就禁用虚拟环境吧）

- 安装mavros并下载地理信息数据

bash

sudo apt install ros-humble-mavros ros-humble-mavros-extras sudo /opt/ros/humble/lib/mavros/install_geographiclib_datasets.sh

- 创建状态机代码的ROS2工作环境

bash

# 创建工作空间 mkdir -p ~/ws_offboard/src cd ~/ws_offboard/src # 创建功能包（包含依赖 mavros_msgs, geometry_msgs, rclcpp） ros2 pkg create offboard_control --build-type ament_cmake --dependencies rclcpp geometry_msgs mavros_msgs

2.1编写状态机代码

在~/ws_offboard/src/offboard_control/src/文件夹下创建一个cpp代码，名字为offb_node.cpp，以下是代码内容：

cpp

/** * @file offb_node.cpp * @brief Pure takeoff solution 鈥?no setpoint during takeoff to avoid conflict. */ #include <rclcpp/rclcpp.hpp> #include <geometry_msgs/msg/pose_stamped.hpp> #include <mavros_msgs/msg/state.hpp> #include <mavros_msgs/srv/command_bool.hpp> #include <mavros_msgs/srv/set_mode.hpp> #include <mavros_msgs/srv/command_tol.hpp> #include <nav_msgs/msg/odometry.hpp> #include <chrono> #include <memory> #include <string> #include <vector> #include <cmath> using namespace std::chrono_literals; enum class MissionState { WAITING_FCU, SETTING_GUIDED, ARMING, TAKEOFF, FLYING_WAYPOINTS, LANDING, DONE }; class OffboardControl : public rclcpp::Node { public: OffboardControl() : Node("offb_node") { // QoS compatible with mavros rclcpp::QoS odom_qos = rclcpp::QoS(rclcpp::KeepLast(10)).best_effort(); rclcpp::QoS state_qos = rclcpp::QoS(rclcpp::KeepLast(10)).best_effort(); state_sub_ = this->create_subscription<mavros_msgs::msg::State>( "/mavros/state", state_qos, std::bind(&OffboardControl::state_cb, this, std::placeholders::_1)); local_pos_sub_ = this->create_subscription<nav_msgs::msg::Odometry>( "/mavros/local_position/odom", odom_qos, std::bind(&OffboardControl::local_pos_cb, this, std::placeholders::_1)); setpoint_pub_ = this->create_publisher<geometry_msgs::msg::PoseStamped>( "/mavros/setpoint_position/local", rclcpp::QoS(10).reliable()); arming_cli_ = this->create_client<mavros_msgs::srv::CommandBool>("/mavros/cmd/arming"); set_mode_cli_ = this->create_client<mavros_msgs::srv::SetMode>("/mavros/set_mode"); takeoff_cli_ = this->create_client<mavros_msgs::srv::CommandTOL>("/mavros/cmd/takeoff"); land_cli_ = this->create_client<mavros_msgs::srv::CommandTOL>("/mavros/cmd/land"); timer_ = this->create_wall_timer(50ms, std::bind(&OffboardControl::control_loop, this)); current_target_ = {0.0, 0.0, 0.0}; publish_setpoint_enabled_ = true; RCLCPP_INFO(this->get_logger(), "Offboard node started (pure takeoff mode)"); } private: void state_cb(const mavros_msgs::msg::State::SharedPtr msg) { current_state_ = *msg; } void local_pos_cb(const nav_msgs::msg::Odometry::SharedPtr msg) { current_altitude_ = msg->pose.pose.position.z; current_position_.x = msg->pose.pose.position.x; current_position_.y = msg->pose.pose.position.y; } void publish_setpoint(double x, double y, double z) { if (!publish_setpoint_enabled_) return; geometry_msgs::msg::PoseStamped sp; sp.header.stamp = this->now(); sp.header.frame_id = "map"; sp.pose.position.x = x; sp.pose.position.y = y; sp.pose.position.z = z; sp.pose.orientation.w = 1.0; setpoint_pub_->publish(sp); current_target_ = {x, y, z}; } void call_set_mode(const std::string& mode) { if (!set_mode_cli_->service_is_ready()) return; auto req = std::make_shared<mavros_msgs::srv::SetMode::Request>(); req->custom_mode = mode; auto future = set_mode_cli_->async_send_request(req); pending_set_mode_future_ = future.future.share(); RCLCPP_INFO(get_logger(), "Requested mode: %s", mode.c_str()); } void call_arm(bool arm) { if (!arming_cli_->service_is_ready()) return; auto req = std::make_shared<mavros_msgs::srv::CommandBool::Request>(); req->value = arm; auto future = arming_cli_->async_send_request(req); pending_arm_future_ = future.future.share(); RCLCPP_INFO(get_logger(), "Requested arming: %s", arm ? "true" : "false"); } void call_takeoff(float alt) { if (!takeoff_cli_->service_is_ready()) { RCLCPP_WARN(get_logger(), "Takeoff service not ready"); return; } auto req = std::make_shared<mavros_msgs::srv::CommandTOL::Request>(); req->altitude = alt; auto future = takeoff_cli_->async_send_request(req); pending_takeoff_future_ = future.future.share(); RCLCPP_INFO(get_logger(), "Takeoff command sent to %.1f m", alt); } void call_land() { if (!land_cli_->service_is_ready()) return; auto req = std::make_shared<mavros_msgs::srv::CommandTOL::Request>(); land_cli_->async_send_request(req); RCLCPP_INFO(get_logger(), "Land command sent"); } void control_loop() { if (publish_setpoint_enabled_) { publish_setpoint(current_target_.x, current_target_.y, current_target_.z); } if (pending_set_mode_future_.valid() && pending_set_mode_future_.wait_for(0s) == std::future_status::ready) { auto result = pending_set_mode_future_.get(); if (result->mode_sent) { RCLCPP_INFO(get_logger(), "GUIDED mode confirmed"); guided_mode_confirmed_ = true; } else { RCLCPP_WARN(get_logger(), "Set mode failed, will retry"); } pending_set_mode_future_ = {}; } if (pending_arm_future_.valid() && pending_arm_future_.wait_for(0s) == std::future_status::ready) { auto result = pending_arm_future_.get(); if (result->success) { RCLCPP_INFO(get_logger(), "Arming confirmed"); armed_confirmed_ = true; current_target_ = {0.0, 0.0, TAKEOFF_ALT}; RCLCPP_INFO(get_logger(), "Setpoint target set to (0,0,%.1f) (will be published after takeoff)", TAKEOFF_ALT); } else { RCLCPP_WARN(get_logger(), "Arming failed, will retry"); } pending_arm_future_ = {}; } if (pending_takeoff_future_.valid() && pending_takeoff_future_.wait_for(0s) == std::future_status::ready) { auto result = pending_takeoff_future_.get(); if (result->success) { RCLCPP_INFO(get_logger(), "Takeoff command accepted by FCU"); } else { RCLCPP_WARN(get_logger(), "Takeoff command rejected"); } pending_takeoff_future_ = {}; } switch (mission_state_) { case MissionState::WAITING_FCU: if (current_state_.connected) { RCLCPP_INFO(get_logger(), "FCU connected"); mission_state_ = MissionState::SETTING_GUIDED; } break; case MissionState::SETTING_GUIDED: if (current_state_.mode == "GUIDED") { guided_mode_confirmed_ = true; } if (!guided_mode_confirmed_ && !pending_set_mode_future_.valid()) { call_set_mode("GUIDED"); } else if (guided_mode_confirmed_) { mission_state_ = MissionState::ARMING; } break; case MissionState::ARMING: if (current_state_.armed) { armed_confirmed_ = true; } if (!armed_confirmed_ && !pending_arm_future_.valid()) { call_arm(true); } else if (armed_confirmed_) { mission_state_ = MissionState::TAKEOFF; } break; case MissionState::TAKEOFF: { static bool takeoff_sent = false; publish_setpoint_enabled_ = false; if (!takeoff_sent) { call_takeoff(TAKEOFF_ALT); takeoff_sent = true; takeoff_start_time_ = this->now(); } if (current_altitude_ >= TAKEOFF_ALT * 0.95) { RCLCPP_INFO(get_logger(), "Takeoff reached target altitude (%.2f m)", current_altitude_); publish_setpoint_enabled_ = true; mission_state_ = MissionState::FLYING_WAYPOINTS; takeoff_sent = false; waypoint_index_ = 0; last_waypoint_time_ = this->now(); current_target_ = {0.0, 0.0, TAKEOFF_ALT}; } else if ((this->now() - takeoff_start_time_).seconds() > 60.0) { RCLCPP_WARN(get_logger(), "Takeoff timeout (60s), forcing to waypoints"); publish_setpoint_enabled_ = true; mission_state_ = MissionState::FLYING_WAYPOINTS; takeoff_sent = false; } break; } case MissionState::FLYING_WAYPOINTS: { const std::vector<std::array<double, 2>> waypoints = { {0.0, 0.0}, {5.0, 0.0}, {5.0, 5.0}, {0.0, 5.0}, {0.0, 0.0} }; double z = TAKEOFF_ALT; if (waypoint_index_ < waypoints.size()) { double target_x = waypoints[waypoint_index_][0]; double target_y = waypoints[waypoint_index_][1]; current_target_ = {target_x, target_y, z}; double dx = current_position_.x - target_x; double dy = current_position_.y - target_y; double dist = std::hypot(dx, dy); if (dist < 0.5) { if ((this->now() - last_waypoint_time_).seconds() > 2.0) { RCLCPP_INFO(get_logger(), "Waypoint %zu reached", waypoint_index_+1); waypoint_index_++; last_waypoint_time_ = this->now(); } } else { last_waypoint_time_ = this->now(); } } else { mission_state_ = MissionState::LANDING; } break; } case MissionState::LANDING: { static bool land_sent = false; if (!land_sent) { call_land(); land_sent = true; current_target_ = {current_position_.x, current_position_.y, 0.0}; } if (current_altitude_ < 0.2) { RCLCPP_INFO(get_logger(), "Landed, mission complete"); mission_state_ = MissionState::DONE; } break; } case MissionState::DONE: RCLCPP_INFO(get_logger(), "Shutting down"); rclcpp::shutdown(); break; } } rclcpp::Subscription<mavros_msgs::msg::State>::SharedPtr state_sub_; rclcpp::Subscription<nav_msgs::msg::Odometry>::SharedPtr local_pos_sub_; rclcpp::Publisher<geometry_msgs::msg::PoseStamped>::SharedPtr setpoint_pub_; rclcpp::Client<mavros_msgs::srv::CommandBool>::SharedPtr arming_cli_; rclcpp::Client<mavros_msgs::srv::SetMode>::SharedPtr set_mode_cli_; rclcpp::Client<mavros_msgs::srv::CommandTOL>::SharedPtr takeoff_cli_; rclcpp::Client<mavros_msgs::srv::CommandTOL>::SharedPtr land_cli_; rclcpp::TimerBase::SharedPtr timer_; mavros_msgs::msg::State current_state_; double current_altitude_ = 0.0; struct { double x=0, y=0; } current_position_; MissionState mission_state_ = MissionState::WAITING_FCU; struct Target { double x=0, y=0, z=0; } current_target_; bool publish_setpoint_enabled_ = true; const double TAKEOFF_ALT = 2.0; size_t waypoint_index_ = 0; rclcpp::Time last_waypoint_time_; rclcpp::Time takeoff_start_time_; std::shared_future<std::shared_ptr<mavros_msgs::srv::SetMode::Response>> pending_set_mode_future_; std::shared_future<std::shared_ptr<mavros_msgs::srv::CommandBool::Response>> pending_arm_future_; std::shared_future<std::shared_ptr<mavros_msgs::srv::CommandTOL::Response>> pending_takeoff_future_; bool guided_mode_confirmed_ = false; bool armed_confirmed_ = false; }; int main(int argc, char **argv) { rclcpp::init(argc, argv); auto node = std::make_shared<OffboardControl>(); rclcpp::spin(node); rclcpp::shutdown(); return 0; }

以上状态机代码执行的效果是：无人机先起飞，再在（与地面平行的）平面内飞了一个矩形，最后降落回起飞点。

因为我的环境有两个缺陷：执行mode guided以后有可能不进入GUIDED模式；执行arm throttle以后有可能解锁失败，即使成功也有可能快速又锁上。状态机代码针对以上两点做了相应的处理：多次执行mode guided直到进入GUIDED模式，多次执行arm throttle，只要检测到解锁成功立马执行takeoff指令（防止重新被锁上）。

希望有人可以改进我的环境配置方法，进而解决这两个缺陷

2.2修改编译配置相关文件

编辑~/ws_offboard/src/offboard_control下的 CMakeLists.txt和 package.xml

CMakeLists.txt

cmake_minimum_required(VERSION 3.8) project(offboard_control) if(CMAKE_COMPILER_IS_GNUCXX OR CMAKE_CXX_COMPILER_ID MATCHES "Clang") add_compile_options(-Wall -Wextra -Wpedantic) endif() # find dependencies find_package(ament_cmake REQUIRED) find_package(rclcpp REQUIRED) find_package(geometry_msgs REQUIRED) find_package(mavros_msgs REQUIRED) find_package(nav_msgs REQUIRED) if(BUILD_TESTING) find_package(ament_lint_auto REQUIRED) # the following line skips the linter which checks for copyrights # comment the line when a copyright and license is added to all source files set(ament_cmake_copyright_FOUND TRUE) # the following line skips cpplint (only works in a git repo) # comment the line when this package is in a git repo and when # a copyright and license is added to all source files set(ament_cmake_cpplint_FOUND TRUE) ament_lint_auto_find_test_dependencies() endif() ament_package() add_executable(offb_node src/offb_node.cpp) target_include_directories(offb_node PUBLIC $<BUILD_INTERFACE:${CMAKE_CURRENT_SOURCE_DIR}/include> $<INSTALL_INTERFACE:include/offboard_control>) target_compile_features(offb_node PUBLIC c_std_99 cxx_std_17) ament_target_dependencies(offb_node rclcpp geometry_msgs mavros_msgs nav_msgs ) install(TARGETS offb_node DESTINATION lib/${PROJECT_NAME})

package.xml

<?xml version="1.0"?> <?xml-model href="http://download.ros.org/schema/package_format3.xsd" schematypens="http://www.w3.org/2001/XMLSchema"?> <package format="3"> <name>offboard_control</name> <version>0.0.0</version> <description>TODO: Package description</description> <maintainer email="zxx@todo.todo">zxx</maintainer> <license>TODO: License declaration</license> <buildtool_depend>ament_cmake</buildtool_depend> <depend>rclcpp</depend> <depend>geometry_msgs</depend> <depend>mavros_msgs</depend> <depend>nav_msgs</depend> <test_depend>ament_lint_auto</test_depend> <test_depend>ament_lint_common</test_depend> <export> <build_type>ament_cmake</build_type> </export> </package>

2.3开始编译

bash

cd ~/ws_offboard colcon build --packages-select offboard_control source install/setup.bash

编译成功以后终于可以开始运行状态机代码了

3运行状态机代码

开启终端1：运行gazebo（以下命令行中的export是针对Nvidia显卡，详情请看《基于ardupilot的gazebo仿真环境配置教程.docx》）

bash1

export __NV_PRIME_RENDER_OFFLOAD=1 export __GLX_VENDOR_LIBRARY_NAME=nvidia export __VK_LAYER_NV_optimus=NVIDIA_only gz sim -v4 -r ~/ardupilot_gazebo/worlds/iris_runway.sdf

正常运行结果如下图

开启终端2：运行飞控ardupilot

bash2

cd ~/ardupilot/Tools/autotest python3 ./sim_vehicle.py -v ArduCopter -f gazebo-iris --model JSON --map --console --out=udp:127.0.0.1:14550

正常运行结果：

应该会出现如图三个窗口：Map（世界地图），Console(临时控制台，里面是你的状态机代码对应的输出)，ArduCopter

（如果发现Map里没有出现四翼无人机标志，请耐心等待）

开启终端3：运行mavros

bash3

ros2 run mavros mavros_node --ros-args -p fcu_url:=udp://:14550@

可能会出现关于mavros.distance_snesor的报错，这是一个有待解决的问题，但这并不影响运行状态机代码

开启终端4：运行状态机代码

bash4

source ~/ws_offboard/install/setup.bash ros2 run offboard_control offb_node

如果一切顺利，你应该能看到无人机先起飞，再在（与地面平行的）平面内飞了一个矩形，最后降落回起飞点。